Some frequently asked questions about DisConnect are discussed below.

1. What are the operating systems that support the DisConnect?

DisConnect has been developed and tested for Linux (Unix) and Windows (XP/Vista/7). System requirement is Perl 5.10 and above (you can download and install free Activeperl community edition for Windows from http://www.activestate.com/activeperl/downloads or http://www.perl.org/get.html; the program is tested with Activeperl community edition 5.12.4.1205).

- What is the typical work flow for determining disulfide connectivity using DisConnect?

Figure 2 shows the typical work flow for determining disulfide connectivity through mass spectrometric fragmentation of the intact molecule, using the DisConnect. Depending upon the size of the polypeptides, it can either be directly fragmented under CID conditions or sized down to a fragmentable scale through an initial proteolysis. Then probable structures for the obtained fragment ions, generated through either mass spectrometric or proteolytic cleavage, are computed through DisConnect; the input being the m/z values of the fragment ions and the sequence of the polypeptide. In case of initial proteolysis, the proteolytic fragments containing Cys residues are further investigated. While the connectivities in peptides that contain a single cystine (disulfide bonded Cys pair) are evident from the MS profile alone, those with multiple cystines are subjected to further CID fragmentation. While analyzing the MS2 data, there may be cases where multiple structures are proposed for a particular fragment ion. Such ambiguities can be resolved through the MS3 fragmentation of the ion. Query the m/z values of these MS3 fragments ions against multiple structures of the parent MS2 ion. At this stage, the structure of the MS2 ion that can explain i.e., shows a hit for all the MS3 ions is considered. This means that it is not chemically possible to obtain all the queried MS3 fragment ions from the screened off structures. The structure that has the correct chemical framework to generate all the queried MS3 fragments filters through. However, it is worth emphasizing that only a set of fragment ions are key to the determination of disulfide connectivity, thus necessitating a judicious choice of the fragment ions to be traced. Hence, it is not essential to determine the structure of every fragment ion with absolutely certainty for connectivity determination. If none of the existing MS2 ions carry enough information to decipher the disulfide framework then similar analysis can carried out on the successive MS3 / MSn fragment ions. The structure of these higher generation fragment ions can be determined by querying their m/z against their corresponding parent fragment ion structures, respectively.

- What is the m/z input format?

For Proteolytic digest analysis:

Before starting DisConnect, Put the m/z values, in the format 'm/z value'space'charge state' (e.g. 1020.2 3, where the m/z value is 1020.2 and charge state 3) in the file 'peak_mass_ms'.

For MSn analysis: Put the m/z values of the fragment ions, in the format 'm/z value'space'charge state' (e.g. 1020.2 3, where the m/z value is 1020.2 and charge state 3) in the file 'peak_mass_ms2' (for MS2) or 'peak_mass_msn' (for MSn).

- What is the sequence input format?

For Proteolytic digest analysis: Paste the protein sequence, ending with a * symbol, in the file "prot_seq".

For MSn analysis: Polypeptide sequence can be entered using the standard one letter codes. For peptide containing multiple chains, like insulin, a letter X should be entered between the two chains (e.g if a peptide conatins two polypeptide chains with sequence GVCSF and RLTCY then the input is GVCSFXRLTCY). For user benefit, if these peptides are results of proteolytic digestion then an input file in the Result_MS folder is created, named "inp_MSn_match_MS_protein name.out" that contains the peptide sequence in the required format for the MS2 analysis. The user can copy the respective sequence from there and paste it. During MS3/MSn analysis, the parent MS2/ MSn-1 ion structure must be in the format as given in the input file in Result_MS2 folder, termed as ‘input_for_MSn_rigorous/smart_Entered complete/fragment sequence’. The user should copy the MSn parent ion (the ion undergoing fragmentation) sequence from this file.

5. What does the sign ‘|’ in the output means?

The sign ‘|’ separates two peptide segments that are not connected through peptide bond linkages. This also means that there must be at least one disulfide bond holding these two discontinuous stretches of peptide segments together. This implies that if there is a structure with two discontinuous peptide segments containing one cysteine each, then these two cysteine residues have to be disulfide linked.

e.g.: A fragment ion with structure GVC1 | NHIC3 means that C1 must be disulfide linked to C3.

6. Can the allotted residue masses for Cys residues for a structure be seen?

Yes. Cysteine residues can take up four possible residue masses (69,135,101,103) (Supplementary Fig. 13), as per the two probable pathways discussed elsewhere. The masses allotted for the Cys residues in a structure are provided along with the structure. Panels from left to right represent the mass allotments for dehydroalanine (69 Da), cysteinpersulfide (135Da), cysteine thioaldehyde (101 Da) and cysteine (103 Da), respectively. If a residue mass is allotted twice, then the sum of it is printed. For example if the residue mass 69Da is allotted twice then 138 will be printed.

7. What is the difference between ‘Smart’ and ‘Rigorous’ modes?

The multiplicity for given m/z largely arises because of a permutation of four different residue mass allotments for Cys residues (see Q5). Although an exclusive presence of pathway A (Supplementary Fig. 13) generating dehydroalanine (69Da) and cysteinpersulfide (135Da) is observed for cystine fragmentation, in positive ion CID conditions, to generalize the program, all the four probable residue masses of Cys (arising from two pathways) have been considered (‘Rigorous’ mode). Smart mode only considers pathway A, which is exclusively observed in positive ion CID conditions. We advise the users to work on ‘smart’ mode while analyzing positive ion CID data.

8. Why certain outputs do not have Cys residues mass allotment printed, in spite of having Cys in the sequence?

For any fragment that contains all the Cys residues that is present in the entire peptide sequence, the possible Cys residue masses are not iterated. The reason is that any such combination eventually leads to sum of residue masses of all the disulfide bonded Cys residues. For example if there are four Cys residues in a peptide then for a fragment containing all the four cysteines, sum of any combination of residue mass arrangement will give rise to 408 (4x102) Da. In this case, the Cys residue masses are not iterated. Also, in some cases the chemical structure of the fragment requires certain numbers of Cys to be disulfide bonded (a fragment having n numbers of peptide chains must be having 2(n-1) numbers of disulfide bonds holding them together, Q9). The iteration is performed for the rest of the Cys residues.

9. In a multi-chain fragment, why all the Cys residues are not iterated?

In general if a structure has n number of peptide chains, then 2(n-1) cysteine residues have to be disulfide linked to hold the n fragments together. Hence these cysteines can not be treated as free and their residue masses will always be 102Da. Hence the iteration of cysteine residue masses should be done only on the rest of the cysteines.

10. Can unnatural amino acid be used with DisConnect?

Yes, we have already included hydroxyproline (O) and pyroglutamic acid (Z). There are three additional letters B, J, U that has not been assigned any residue masses. Any amino acid, other than the 22 already listed, can be incorporated in all the residue mass list files within the folder Source code (name beginning with mass_list_) of the program.

11. Can the neutral losses be analyzed?

Yes. Typically the fragment ions arising from the neutral losses are present as a subset of the precursor fragment ion. So once the structure of the major fragment ion is known, the ion arising through neutral losses can be easily assigned. The problem arises when neutral loss ions are exclusively present with less or no abundance for the precursor fragment. This program has been built to also directly analyze the fragment in these cases. If no structure is found for an m/z, then different neutral loss pathway can also be searched. A point to note that the output structure is the structure of the parent fragment ion from which the particular neutral loss has taken place.